NYSCTU Focused Reconnaissance Assessment Protocol: Field Data Sheet

Date:_____________  Party: __________________________________________________________________________________________

Chapter: ________________________________________________  #_____________   yrly rainfall: ________ DEC region:__________

County:__________________________________Township:_________________________________________ valley type (I-IX): ______

Watershed code:_______ Stream: ___________________________________________HUC code: ______________________________

Reach:  ________________________________________________________________    Active channel width: ____________________

Length:______________________ ft             Bkfl Width:______________ft                       Active Flood Plain Width:-_________________

HUC code: _____________________________________________               Coord:_____________________N ____________________W

Drainage Area _____________ mi sq     Dominant substrate: ______(1-6)    Channel form _______ (Aa-G)     Reference reach:  Y /  N /  ?

- Channel Stability Indicators:  circle responses & criteria (from Vt. ANR)

Aggredation evidence:   Y  /  N

1	riffle embedded			2	pool siltation

3	mid channel bars & braiding		4	growing bars, no vegetation

5	bar W.>½ WSW at low flow		6	point bar particle much coarser than riffle particles

Degredation evidence:   Y /   N

1	exposed infrastructure		2 	undermined bank revetment

3	cut faces on point bars		4	head cuts, over steep riffle

5	suspended armor layer		6	large unvegetated point bars with much finer

than riffle particles (see RSI)

Widening evidence:     Y  /   N

1	undercut leaning trees		2	exposed tree roots

3	toe scour on inside of meanders	4	toe scour on both sides of riffle

5	non cohesive soil in steep bank		6	lack of vegetation at eroding banks

Planform adjustments:   Y  /  N

1	channel braids, flood chutes, cutoffs	2	runs replace riffle/pool features

3	thalweg out of phase			4	aerial photo (meander form, ROC)

- Abbreviated Cross Section (set tape at bankfull elevation, note optional bkfl candidates)

LFPL	        LBKFL          opt          LWS          opt           Thlwg           opt             RWS         RBKFL          opt          Top Low Bk             RFP

sta:______________________________________________________________________________________________________________

rod:______________________________________________________________________________________________________________

Bankfull Width:________________	Flood Prone Width:______________	Entrenchment Ratio (FPW/Wbkfl):___________________

Cross Sectional Area:_____________ 	Bank Height Ratio (Low Bank Ht/Bankfull Height): ________________  BHR:  ok  slight   mod    extreme

- Bank Erodability Hazard Index:  a quick semi quantitative measure of bank erosion risk  (see Rosgen fig. 6.25)

visually estimate or,  using a marked staff,  approximate:

value		index

- Bank Ht/ Bkfl Ht      ____________	             ________			Final BEHI index (see sheet) ______________________

- Root depth/Bank Ht ____________	             ________

- Root Density (%)     ____________	             ________			BEHI Interpretation:	 vl	      low	        mod

(circle)

- Bank Angle             ____________	             ________					hi	     vhi	        extreme

- % Surface protect.  ____________	             ________

- Near Bank Stress Index  (visual approximation or quantitatively from cross section;  see Rosgen fig. D-8)

- Area Near Bank/ total XC area  OR   Near Bank mean depth/ total XC mean depth ~= NB shear stress/ total XC shear stress

value:________________________		NBSI   Interpretation:   .32 or <            .33-.41	  .42-.45	        .46-.5                >.5

(circle)           low	            mod	      hi	           vhi		ext

- Riffle Stability Index:  	Riffle D50: ___________mm               D70__________mm	      D90__________mm

mean largest particle size on point bar: ______+ _______+_______+______+______+_______+______+_______+______+______ /n = ____

RSI interpretation:    	<50			50-70			70-90			90-100

(circle)                               downcutting		                stable		             transitional		              aggrading