Geomorphic Cluster

Channel Condition. natural channel no structures, no berms, no downcutting, no lateral cutting; past channel alteration, recovered channel recovered banks no or well set back berms providing access to floodplain; altered channel < 50% riprap or channelized excess aggradation constricting berms (or) braided channel; severely altered channel > 50% rip rap or channelized active downcutting; active widening berms block floodplain access (one or both banks). 10; 7; 3; 1. Hydrologic Alteration. over bank flooding = 1½ - 2 years no channel incision no water withdrawals no dams full access to flood plain; over bank flooding = 3 - 5 years limited channel incision (or) minor water withdrawal not affecting available biota habitat; over bank floods = 6 - 10 years channel deeply incised (or) significant water withdrawal affects biota habitat during low flow; no over bank flooding deeply incised structures block access to floodplain (or) severe loss of low flow habitat from water withdrawal (or) flooding yearly or more frequent. 10; 7; 3; 1. Deviation from Bankfull Width (WBKFL). none channel at bankfull width +/- < 10%; channel 10 - 20% greater than expected bankfull width; channel > 20% but < 50% of expected bankfull width; channel > 50% greater than expected bankfull width. 10; 7; 3; 1. Bank Stability. stable Bank Height = Bankfull height > 33% of eroding surface on outer bends protected by roots extending to base flow; moderately stable Bank height = Bankfull height < 33% of eroding surface on outer bends protected by roots extending to base flow; moderately unstable Bank height > 1½x Bankfull height active erosion on outside bends (overhang, annual tree loss, some slope failure) banks typically high; unstable Bank Height ≥ 2x Bankfull height active erosion on straight or inside bends with numerous slope failures, tree loss annually, trees absent. 10; 7; 3; 1.

Fisheries Cluster

Instream Fish Cover; Cover types: LWD, deep pools, overhanging vegetation, boulders/large cobble, deep riffles, undercut banks, thick root mats, dense macrophyte beds, isolated/backwater pools. >7 cover types; 6 - 7 cover types; 4 - 5 cover types; 2 - 3 cover types; 0 - 1 cover type. 10; 8; 5; 3; 1. Riparian Zone. natural vegetation extends > 2 bankfull widths on each side; natural vegetation extends ≈ 1 - 2 bankfull widths on each side; natural vegetation extends ≈ ½ bankfull width on each side; natural vegetation extends ≈ ⅓ bankfull width on each side (or) filtering function moderately compromised; natural vegetation extends < ⅓ bankfull width on each side, (or) lack of regeneration (or) filtering function severely compromised. 10; 8; 5; 3; 1. Pool Status. deep pools common 5 feet or more at base flow; present, not abundant depth ≥ 3 feet at base flow; present, shallow depth < 3 feet at base flow; absent or very shallow. 10; 7; 3; 1. Canopy. > 75% shaded in reach and upstream 2 - 3 miles well shaded; > 50% shaded in reach (or) > 75% in reach but upstream 2 - 3 miles poorly shaded; 20 - 50% shaded in reach; < 20% shaded in reach. 10; 7; 3; 1. Fish Barriers. none; seasonal withdrawals inhibit movement; obstructing vegetative debris within reach; structures < 1’ drop within reach; structures > 1’ drop within 3 miles; structures > 1’ drop within reach. 10; 8; 7; 5; 3; 1.

Water Quality Cluster

Water Appearance. very clear or tea colored objects visible at 3’ - 6’ no oil sheen no film on submerged objects or rocks; occasionally cloudy, clears rapidly objects visible at 1½’ - 3’ may be slightly green no oil sheen; mostly cloudy objects visible at ½‘ - 1 ½’ pools may be pea green submerged objects with green film (or) odor of ammonia or rotten eggs; very turbid usually objects visible at < ½’ pools bright green obvious pollutants, scum, sheen, foam, floating algal mats (or) strong odor of pollutants. 10; 7; 3; 1. Nutrient Enrichment. clear along entire reach diverse aquatic plants low quantities of many macrophytes little algal growth; fairly clear or slightly greenish moderate algal growth on substrates; greenish water lush green macrophytes abundant algal growth especially in warmer months; pea green, gray or brown clogging macrophytes severe algal blooms thick algal mats. 10; 7; 3; 1. Temperature Range. optimal year round for salmonid species present; summer or winter temperatures rarely & briefly in stress range no effect on population; temperatures frequently reach stress range, thermal kills occur once every 5 - 10 years, periodic impacts on population; sustained major thermal stress occurs at least annually, population depressed due to thermal effects. 10; 7; 3; 1. Manure Presence *. none; evidence of livestock access to riparian zone; occasional in stream/on banks or waste storage on floodplain; extensive on banks or in stream (or) untreated sewage discharge. No Score; 5; 3; 1.

Invertebrate Cluster

Insect/Invertebrate Habitat; Habitat types: fine woody debris, submerged logs, leaf packs, undercut banks, coarse gravel, cobble, boulders. ≥ 5 habitat types, at correct stage for colonization; 3 - 4 habitat types, some potential habitat; 1 - 2 habitat types, substrate covered or scoured at high flows; 0 - 1 habitat type. 10; 7; 3; 1. Riffle Embeddedness *. gravel/cobble < 20% embedded; gravel/cobble 20 - 30% embedded; gravel/cobble 30 - 40% embedded; gravel/cobble > 40% embedded; gravel/cobble in riffle completely embedded. 10; 8; 5; 3; 1. Macroinvertebrates *. Group I with good diversity (mayflies, stoneflies, caddis, hellgrammites); Group II dominates (damselflies, dragonflies, sowbugs, blackflies, crayfish); Group III dominates (midges, craneflies, horseflies, leeches, worms); very reduced number of species or near absence of all macroinvertebrates. 15; 6; 2; -3.

| * score only if applicable |  |  | Channel Forms |  | Entrenchment | W/D | Sinuosity | Slope |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Aa+ | Very steep, deeply entrenched, torrent streams | < 1.4 | < 12 | 1.0 – 1.1 | > .10 |
| Dominant Substrate Material |  |  | A | Steep, entrenched, cascading, step/pool streams | < 1.4 | < 12 | 1.0 – 1.2 | .04 - .10 |
|  |  |  | B | Moderately entrenched, moderate gradient, riffle dominated channel with infrequently spaced pools | 1.4 – 2.2 | > 12 | > 1.2 | .02 - .039 |
| 1 = 2 = 3 = 4 = 5 = 6 = | bedrock boulders cobble gravel sand silt/clay |  | C | Low gradient, meandering, point-bar, riffle/pool, alluvial channels with broad well defined floodplains | > 2.2 | > 12 | > 1.4 | < .02 |
|  |  |  | D | Braided channel with longitudinal & transverse bars | n/a | > 40 | n/a | < .04 |
|  |  |  | DA | Anastamosing narrow & deep with extensive, well vegetated floodplains & associated wetlands | > 2.2 | Highly variable |  | < .005 |
|  |  |  | E | Low gradient, meandering riffle/pool stream with low width/depth ration & little deposition | > 2.2 | < 12 | > 1.5 | < .02 |
|  |  |  | F | Entrenched, meandering riffle/pool channel on low gradients with high width/depth ratio | < 1.4 | > 12 | > 1.4 | < .02 |
|  |  |  | G | Entrenched “gully” step/pool and low width/depth ratio on moderate gradients | < 1.4 | < 12 | > 1.2 | .02 - .039 |

| Date: | / / |  |  |  |  |  |  |  |  | Survey Party: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Chapter: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | # |  |  |  |  |  |  |  |  |  | DEC Region: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| County: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Township: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Watershed Code: |  |  |  |  |  |  |  |  |  |  |  | Watershed Index # (HUC): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Stream: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reach: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Coord’s: |  |  |  |  |  |  |  |  |  |  |  |  |  |  | N |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | W |  |  |  | Drainage Area: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | sq. mi. |  |  |
| Reach Length: |  |  |  |  |  |  |  |  |  |  |  |  | Ft. |  |  |  | Active Floodplain Width: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Ft. |  |  |  |  | Bankfull Width: |  |  |  |  |  |  |  |  |  |  |  |  |  | Ft. |
| Dominant Substrate: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (1 – 6) |  |  |  |  |  | Channel Form: |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | (Aa+ - G) |  |  |  |  |  |  |  |  | Valley Type: |  |  |  |  |  |  |  |  |  |  | (I – IX) |  |
| Land Use in Drainage Area (%): |  |  |  |  |  |  |  |  |  |  |  |  |  | Row crop |  |  |  |  |  |  |  |  |  | Hayland |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Grazing |  |  |  |  |  |  |  |  |  |  |  | Forest |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Confined animal feeding |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Conservation reserve |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  |  |  |  |  |  |  | Industrial/Commercial |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Residential |  |  |  |  |  |  |  |  |  |  |  |  |  |  | Other |  |  |  |  |  |  |  |

| See protocol & field book summary sheet for specific criteria. |  |  |  | Rankings (weighted 15 to -3 scale) |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Item |  | Score |  | Optimal Severely impacted |  |  |  |  |  |  |  |  |  |  |  |  |
| Geomorphic cluster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Channel Condition |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Hydrologic Alteration |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Deviation from Wbkfl |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Bank Stability |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Fisheries cluster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Instream Fish Cover |  |  |  |  | 10 |  |  | 8 |  |  | 5 |  | 3 |  | 1 |  |
| Riparian Zone |  |  |  |  | 10 |  |  | 8 |  |  | 5 |  | 3 |  | 1 |  |
| Pool Status |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Canopy |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Fish Barriers |  |  |  |  | 10 |  |  | 8 | 7 |  | 5 |  | 3 |  | 1 |  |
| Water quality cluster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Water Appearance |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Nutrient Enrichment |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Temperature Range |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Manure Presence * |  |  |  |  |  |  |  |  |  |  | 5 |  | 3 |  | 1 |  |
| Invertebrate cluster |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Insect/Invertebrate Habitat |  |  |  |  | 10 |  |  |  | 7 |  |  |  | 3 |  | 1 |  |
| Riffle Embeddedness * |  |  |  |  | 10 |  |  | 8 |  |  | 5 |  | 3 |  | 1 |  |
| Macroinvertebrates * |  |  |  | 15 |  |  |  |  |  | 6 |  |  |  | 2 |  | -3 |
| * NOTE: These items are optional on the Stream Visual Assessment Protocol |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

| Average score for reach: |  |  |  |  | Status: | Poor |  |  | Fair |  | Good |  | Excellent |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | < 6.0 |  |  | 6.1 – 7.4 |  | 7.5 – 8.9 |  | > 9.0 |
| (Total divided by number of elements scored) |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Reference reach candidate: |  | Yes |  |  | No |  |  | Possible |  |  |  |  |  |

| Additional studies needed: |  |
| --- | --- |

| Summary of observed problems: |  |
| --- | --- |

| Suspected causes: |  |
| --- | --- |

| Options/Recommendations: |  |
| --- | --- |

| Field Sketch: |  |
| --- | --- |
|  | Banks identified facing downstream. |